# UNIVERSITY OF GHANA

# DEPARTMENT OF COMPUTER ENGINEERING

# SCHOOL OF ENGINEERING SCIENCES

# COLLEGE OF BASIC AND APPLIED SCIENCES

# SECOND SEMESTER – 2025/2026 ACADEMIC YEAR

# CPEN 438 - Advanced Computer Organization and Architecture

# Course Instructor: Prof. Robert A. Sowah

Project 2:

GhanaCore-5 - Five-Stage Pipelined Processor for MoMo Transaction Codes

Group 7

# WEEKLY COURSE PROJECT REPORT

Course Code and Title: CPEN 438 - Computer Architecture
Project Title: Project 2: GhanaCore-5, Five-Stage Pipelined Processor for MoMo Transaction Codes
Team Number/Name: Group 7
Reporting Week: Week 2				Period: [10/08/26] to [14/08/26]
Team Leader/Reporter: Ahmed Fareed Opare     Submission Date: 19/08/2026

## 1. Weekly Objective and Progress Summary

Main objective for the week:

GhanaCore-5 Week 2 delivers the combinational opcode-to-control mapping and complete truth table for all twelve opcodes, the exact IF/ID, ID/EX, EX/MEM, and MEM/WB pipeline-register field layouts, a dual-read/single-write register file, a golden C simulator with instruction-memory loading and a fetch/decode/execute/memory/writeback dispatch loop, and a two-pass Python assembler that encodes the seeded 18-instruction MoMo routine from Week 1.

Overall project completion: 10%

Summary of progress achieved:

Week 2 plan was to build the combinational control unit, four pipeline registers, the register file, a golden C simulator, and a two-pass assembler.

The combinational control unit was completed in `control_unit.h` and `control_unit.py`, mapping all twelve opcodes to `RegWrite`, `MemRead`, `MemWrite`, `MemToReg`, `ALUSrcImm`, `Branch`, `BranchNE`, and `ALUOp`. The mappings are documented in `docs/control_truth_table.md` and `docs/control_truth_table.csv`.

The four pipeline registers were fully specified as IF/ID (65-bit), ID/EX (157-bit), EX/MEM (145-bit), and MEM/WB (77-bit) in `docs/pipeline_register_spec.md` and `docs/pipeline_register_fields.csv`, with the layout written for direct Logisim construction.

A dual-read/single-write 16-register file was implemented in `simulator/register_file.h`.

The golden C simulator was completed in `simulator/simulator.c`, including the IF-ID-EX-MEM-WB dispatch loop, hex instruction/data-memory loading, and per-cycle CSV export of all registers and sixteen memory words.

The two-pass Python assembler was completed in `assembler/assembler.py`, providing label resolution and encoding for the seeded 18-instruction MoMo routine, with hex, listing, CSV, and Markdown machine-code outputs.

The key Week 2 result is that the assembler and golden simulator agree end-to-end: the accepted transaction finishes with `memory[0]=840` and `r8=1`, while the rejected transaction finishes with `memory[0]=1000` and `r8=0`.

This provides Week 3's Logisim implementation with a verified software reference for cycle-by-cycle comparison.

## 2. Meetings Held

| Date and time | Duration/Mode | Members Present | Main Agenda | Decisions and Outcomes |
| --- | --- | --- | --- | --- |
| 10/08/2026, 7:00 PM | 60 min · Online | Fareed Opare Ahmed, Kwesi Asante Agyepong, Isaac Nii Nortey Barnor, Mohammed Salihu Hamisu | Plan Week 2 build: control unit, pipeline registers, register file, golden C simulator, two-pass assembler | ISA revised to 18 instructions / 12 opcodes with fixed-width R/I/S-B encoding during implementation (down from the 20-mnemonic R/I/J draft approved in the Week 1 review) to simplify the control unit and pipeline-register packing; tasks assigned per Section 3 |
| 14/08/2026, 4:00 PM | 30 min. Online | Fareed Opare Ahmed, Kwesi Asante Agyepong, Isaac Nii Nortey Barnor, Mohammed Salihu Hamisu | Review Week 1 deliverables: draft MoMo ISA, opcode/format table, RISC paper review | All 18 C-core checks and 8 Python tests passing (make test); accepted- and rejected-transaction integration runs verified against hand-computed values; approved for submission; Week 3 to begin the Logisim datapath build from the pipeline-register spec |

## 3. Work Completed and Individual Contributions

| Member | Assigned Task | Work Completed | Evidence/Output | Status |
| --- | --- | --- | --- | --- |
| Mohammed Salihu Hamisu | Control-signal truth table | Implemented the combinational opcode-to-control mapping for all twelve opcodes (RegWrite, MemRead, MemWrite, MemToReg, ALUSrcImm, Branch, BranchNE, ALUOp) as matching C and Python modules, plus the exported Markdown/CSV truth table. | simulator/control_unit.h; simulator/control_unit.py; docs/control_truth_table.md; docs/control_truth_table.csv | Completed |
| Kwesi Asante Agyepong | Pipeline register spec + register file | Wrote the exact bit-field layout for IF/ID (65 bits), ID/EX (157 bits), EX/MEM (145 bits), and MEM/WB (77 bits) as a Logisim build specification with named subcircuits and signals, and implemented a dual-read/single-write 16-register file with a hardwired r0. | docs/pipeline_register_spec.md; docs/pipeline_register_fields.csv; simulator/register_file.h | Completed |
| Ahmed Fareed Opare | Golden C simulator | Built the fetch/decode/execute/memory/writeback dispatch loop, the hex instruction- and data-memory loader, EX-stage branch resolution with a two-instruction flush, and per-cycle CSV export of all sixteen registers and sixteen memory words. | simulator/simulator.c; build/momo_trace.csv | Completed |
| Isaac Nii Nortey Barnor | Two-pass assembler + unit tests | Implemented label-collection and encoding passes in assembler/assembler.py, and unit-tested the register file, control unit, decoder, assembler, and end-to-end pipeline against hand-computed expected values. | assembler/assembler.py; tests/test_core.c; tests/test_control_unit.py; tests/test_assembler.py; tests/test_integration.py; docs/testing_verification_results.md | Completed |

## 4. Work Process and Technical Activities

Methods, tools, and procedures used:

Implemented the control unit twice, in both C (`control_unit.h`) and Python (`control_unit.py`), using the same opcode table and cross-checking both implementations against an independently hand-computed truth table covering every legal opcode plus one reserved opcode.

Specified the four pipeline registers as named Logisim subcircuits — `REG_IF_ID_65`, `REG_ID_EX_157`, `REG_EX_MEM_145`, and `REG_MEM_WB_77` — with exact bit-packed fields so the Hardware Lead can construct them directly.

Completed the golden simulator in C11 using `-Wall -Wextra -Wpedantic -Werror`, with write-before-read register-file timing and EX-stage branch resolution with a two-instruction flush.

Because this baseline has no forwarding, data hazards are handled by inserting NOPs in `programs/momo_routine_nop_padded.s`.

Extended the Week 1 ISA module into a two-pass Python assembler that first collects labels and then encodes instructions, using regular-expression operand parsing and `offset(base)` memory syntax.

Testing, verification, or review conducted:

Ran `make test`, which builds and executes `tests/test_core.c` and the Python test suite under `tests/`.

All tests passed: **18 C-core checks and 8 Python tests**, with zero failures. The tests confirmed that every opcode produces the expected control-signal tuple, the register file supports simultaneous dual reads with a single enabled write and a hardwired `r0`, and the assembler reproduces all 18 Week 1 MoMo machine words exactly.

End-to-end NOP-padded pipeline testing also passed for both transaction paths. The accepted transaction, using a balance of 1000, amount of 150, fee of 10, and limit of 500, produced `memory[0]=840` and `r8=1` after 34 cycles, with a CPI of 1.214. The rejected transaction, with an amount of 600, left `memory[0]=1000` unchanged and produced `r8=0` after 18 cycles, with a CPI of 1.500.

No corrections were required after the first passing run, and the complete results are recorded in `docs/testing_verification_results.md`.

## 5. Challenges, Changes, and Corrective Actions

| Challenge or Deviation | Effect on Project | Action Taken/Proposed | Responsible Member |
| --- | --- | --- | --- |
| Team roster and a Week 2 meeting log are not yet captured in this report. | Individual-contribution attribution and the accountability declaration are incomplete pending team input. | Team to confirm member names, meeting date/time/mode, and sign off before submission. | Ahmed Fareed Opare |
| The baseline pipeline has no forwarding or automatic hazard detection, so the 18-instruction Week 1 routine cannot run correctly as written. | Back-to-back dependent instructions (e.g. an SLT immediately after the LW it depends on) would read stale register values without manual intervention. | Added an explicit NOP-padded baseline program (programs/momo_routine_nop_padded.s, 31 words) alongside the original 18-instruction routine, and documented in README.md and pipeline_register_spec.md that forwarding is deferred; the golden simulator and Logisim build both run the padded version. | Whole team |

## 6. Plan for the Next Week

| Planned Task or Deliverable | Responsible Member(s) | Deadline | Expected Evidence |
| --- | --- | --- | --- |
| Build the Logisim/Digital top-level GhanaCore5 circuit (PC, InstructionROM, ControlUnit, RegisterFile16x32_2R1W, ALU32, DataRAM256x32) from the Week 2 pipeline-register spec | Whole team | Week 3 report deadline (Sun 2026-08-23) | logisim/*.circ datapath circuit |
| Wire the four pipeline-register subcircuits (REG_IF_ID_65, REG_ID_EX_157, REG_EX_MEM_145, REG_MEM_WB_77) and the EX-stage branch-flush logic | Whole team | Week 3 report deadline (Sun 2026-08-23) | logisim/*.circ; probe screenshots |
| Run the NOP-padded MoMo routine through Logisim and compare probe values cycle-by-cycle against build/momo_trace.csv from the golden simulator | Whole team | Week 3 report deadline (Sun 2026-08-23) | Logisim run log vs. simulator CSV trace diff |

.

## 7. Team Accountability Declaration

We confirm that this report accurately represents the meetings, work completed, individual contributions, challenges, and evidence produced during the reporting week.

Team Leader: Ahmed Fareed Opare 				Date: 19/08/2026

Team Members’ Initials: AFO, KAA, INNB, MSH

Evidence repository/link: